262 – Lab 4

Scrolling LED with Speed and Pattern

NAME:	POSSIBLE POINTS: 10

STUDENT ID:

COURSE DATE & TIME:

LAB OBJECTIVE

- Add a second pushbutton switch that modifies the LED pattern.

- Improve the accuracy of hardcoded delay values by measuring timing with an oscilloscope and adjusting accordingly.

OPERATION

A second pushbutton shall be added to the breadboard circuit. This pushbutton will control an alternate LED pattern.

- When the second pushbutton is not pressed, the LED pattern from the previous lab shall operate unchanged.

- When the second pushbutton is held down, the LED pattern shall change to the following:

  - All LEDs are ON except one LED that is OFF.

  - The single OFF LED shall scroll from right to left.

  - When the OFF LED reaches the far left position, the pattern shall reverse direction and scroll from left to right.

  - This alternating left/right scrolling shall continue indefinitely while the button remains pressed.

The oscilloscope shall be used to verify and improve timing accuracy:

- Using the previous lab (Lab 3) code, capture two waveform measurements:

  - Approximately 0.5 s delay and Approximately 1.0 s delay

- Adjust the hardcoded delay values in this lab so that the delays are precisely:

  - 0.5 s and 1.0 s

Use oscilloscope measurements to guide your adjustments.

- After modifying the delay values, capture two additional waveform measurements showing the corrected timing.

A total of four waveform images must be included in the lab write-up:

- Two captures (0.5 s and 1.0 s) before adjustment

- Two captures (0.5 s and 1.0 s) after adjustment

DELIVERABLES

LAB WRITE-UP

The lab write-up must include the following items in a single document submitted on Canvas:

- This document as the cover sheet

- Answers to any assigned questions

- Source code

- A clear Photo of full hardware setup (dev board + breadboard)

- captures:

  - Two captures (0.5 s and 1.0 s) before delay correction

  - Two captures (0.5 s and 1.0 s) after delay correction

  - Each waveform must include vertical cursor measurements showing the ON-time of the LED

DEMONSTRATION

A video demonstration must be submitted in the same Canvas assignment folder.

The video must clearly show:

- Full left-to-right and right-to-left scrolling

- Immediate pattern changes when the second pushbutton is pressed or released

- Immediate speed changes during scrolling (no reset required)

- Both pushbuttons affecting the system simultaneously