262 – Lab 5

Scrolling LED with Speed, Pattern and Timer

NAME:	POSSIBLE POINTS: 10

STUDENT ID:

COURSE DATE & TIME:

LAB OBJECTIVE

- Change the hardcoded for-loop busywait delays to an improved timer based solution.

OPERATION

A Timer will be used now to create the 0.5s and 1s delay timing.

- Timer 0 16-bit mode

- The Timer delay code will live in a function called delay() with the following prototype: void delay(unsigned int ms) where a number of milliseconds gets passed to the function and the timer implements that delay time. Recommendation: use the timer to generate 10ms of delay and then use a loop to perform this 10ms multiple times.

The oscilloscope will be used to verify the improved timing accuracy:

- Using the previous lab (Lab 4) code, capture two waveform measurements:

  - Approximately 0.5 s delay and Approximately 1.0 s delay

Use oscilloscope measurements to verify the new timer based timing.

- After implementing the timer based delay, capture two additional waveform measurements showing the corrected timing.

A total of four waveform images must be included in the lab write-up:

- Two captures (0.5 s and 1.0 s) from last lab before changing to the timer

- Two captures (0.5 s and 1.0 s) after changing to the timer

DELIVERABLES

LAB WRITE-UP

The lab write-up must include the following items in a single document submitted on Canvas:

- This document as the cover sheet

- Answers to any assigned questions

- Source code

- A clear Photo of full hardware setup (dev board + breadboard)

- captures:

  - Two captures (0.5 s and 1.0 s) before timer

  - Two captures (0.5 s and 1.0 s) after timer

  - Each waveform must include vertical cursor measurements showing the ON-time of the LED

DEMONSTRATION

A video demonstration must be submitted in the same Canvas assignment folder.

The video must clearly show:

- Full left-to-right and right-to-left scrolling

- Immediate pattern changes when the second pushbutton is pressed or released

- Immediate speed changes during scrolling (no reset required)

- Both pushbuttons affecting the system simultaneously