NAME:	POSSIBLE POINTS: 10

STUDENT ID:

COURSE DATE & TIME:

OBJECTIVE:

- To parallel interface an HD44780 2x16char LCD to the 8051.

- Adding the LCD to the previous Servo Lab 6.

- To display the current angle that the servo is at on the LCD.

OPERATION:

The LCD should display the current angle that the servo is at. The servo driven to the extreme CCW is considered -45 and maximum CW is +45. Since the servo is being incremented by 4.5deg each step, the LCD should display to one decimal point of precision as well as display the polarity + or –

Examples:

Angle: -45		Angle: -13.5		Angle: +4.5

Angle: +36		Angle: +45

DELIVERABLES

LAB WRITE-UP

The lab write-up must include the following items in a single document submitted on Canvas:

- This document as the cover sheet

- Answers to any assigned questions

- Source code

- Schematic: Hand-Drawn or in a schematic entry tool (Altium, KiCad etc…)

- A clear Photo of full hardware setup (dev board + breadboard)

- Oscilloscope Waveform Captures:

  - 3 Captures at Servo Far Left Angle, Center, Far Right

  - Each waveform must include vertical cursor measurements showing the ON-time of the Pulse

DEMONSTRATION

A video demonstration must be submitted in the same Canvas assignment folder.

The video must clearly show:

- Servo Center at Initial Power with the LCD displaying “Angle: 0”

- 10x Down Button Presses to Far Left followed by

- 20x Up Button Presses to Far Right

- The correct Angle should always be displayed on the servo and should update immediately after a change.